Lab 8

- Leaky bucket

#include <stdio.h>

int min(int x, int y) {

return (x < y) ? x : y;

}

int main() {

int cap, process, nsec;

int inp[25];

int count = 0, drop = 0, mini;

printf("Enter the bucket size:\n");

scanf("%d", &cap);

printf("Enter the processing rate:\n");

scanf("%d", &process);

printf("Enter the number of seconds you want to simulate:\n");

scanf("%d", &nsec);

for(int i = 0; i < nsec; i++) {

printf("Enter the size of the packet entering at %d sec:\n", i+1);

scanf("%d", &inp[i]);

}

printf("\nSecond | Packet Recvd | Packet Sent | Packet Left | Dropped\n");

printf("---------------------------------------------------------------\n");

// Main simulation for given seconds

for(int i = 0; i < nsec; i++) {

count += inp[i];  // packets received

if(count > cap) {

drop = count - cap;

count = cap;

}

mini = min(count, process); // process packets

count -= mini;

printf("%6d | %12d | %11d | %11d | %7d\n", i+1, inp[i], mini, count, drop);

drop = 0;

}

// Flush the remaining packets after time ends

int i = nsec;

while(count > 0) {

if(count > cap) {

drop = count - cap;

count = cap;

}

mini = min(count, process);

count -= mini;

printf("%6d | %12d | %11d | %11d | %7d\n", i+1, 0, mini, count, drop);

drop = 0;

i++;

}

return 0;

}

- Crc

#include <stdio.h>

#include <string.h>

#include <stdlib.h>

int main() {

char rem[50], a[80], s[80], c, msj[50], gen[30];

int i, genlen, t, j, flag = 0, k, n;

printf("Enter the generator polynomial (example: 10001000000100001 for CRC-CCITT):\n");

gets(gen);

printf("Generator polynomial is CRC-CCITT: %s\n", gen);

genlen = strlen(gen);

k = genlen - 1;

printf("Enter the message:\n");

n = 0;

while ((c = getchar()) != '\n' && c != EOF) {

msj[n++] = c;

}

msj[n] = '\0';

// Copy message and append k zeros

for (i = 0; i < n; i++)

a[i] = msj[i];

for (i = 0; i < k; i++)

a[n + i] = '0';

a[n + k] = '\0';

printf("\nMessage polynomial appended with zeros:\n");

puts(a);

// Division to get remainder

for (i = 0; i < n; i++) {

if (a[i] == '1') {

t = i;

for (j = 0; j <= k; j++) {

a[t] = (a[t] == gen[j]) ? '0' : '1';

t++;

}

}

}

// Extract remainder

for (i = 0; i < k; i++)

rem[i] = a[n + i];

rem[k] = '\0';

printf("The checksum (remainder) is:\n");

puts(rem);

printf("\nMessage with checksum appended:\n");

for (i = 0; i < n; i++)

a[i] = msj[i];

for (i = 0; i < k; i++)

a[n + i] = rem[i];

a[n + k] = '\0';

puts(a);

// Receiver side

printf("\nEnter the received message:\n");

n = 0;

while ((c = getchar()) != '\n' && c != EOF) {

s[n++] = c;

}

s[n] = '\0';

// Re-division at receiver

for (i = 0; i < n; i++) {

if (s[i] == '1') {

t = i;

for (j = 0; j <= k; j++, t++) {

s[t] = (s[t] == gen[j]) ? '0' : '1';

}

}

}

// Check remainder at receiver

for (i = 0; i < k; i++)

rem[i] = s[n + i];

rem[k] = '\0';

for (i = 0; i < k; i++) {

if (rem[i] == '1')

flag = 1;

}

if (flag == 0)

printf("Received polynomial is ERROR-FREE\n");

else

printf("Received polynomial contains ERROR\n");

return 0;

}