B.M.S. COLLEGE OF ENGINEERING BENGALURU

Autonomous Institute, Affiliated to VTU

Lab Record

Computer Networks – 23CS5PCCON

Submitted in partial fulfillment for the 5th Semester Laboratory

Bachelor of Engineering

in

Computer Science and Engineering

Submitted by:

DISHA D S

(1BM23CS094)

Department of Computer Science and Engineering

B.M.S. College of Engineering

Bull Temple Road, Basavanagudi, Bangalore 560 019

August 2025-December 2025

B.M.S. College of EngineerinG

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

CERTIFICATE

This is to certify that the Computer Networks (23CS5PCCON) laboratory has been carried out by DISHA D S (1BM23CS094) during the 5th Semester August 2025-December 2025

Signature of the Faculty Incharge:

Prof. Sarala D V

Assistant Professor

Department of Computer Science and Engineering

B.M.S. College of Engineering, Bangalore

Table of Contents

| PART - A |  |
| --- | --- |
| Serial No. | Name of Expirement |
| 1. | Create a topology and simulate sending a simple PDU from source to destination using hub and switch as connecting devices and demonstrate ping message. |
| 2. | Configure DHCP within a LAN and outside LAN. |
| 3. | Configure Web Server, DNS within a LAN. |
| 4. | Configure IP address to routers in packet tracer. Explore the following messages: ping responses, destination unreachable, request timed out, reply. |
| 5. | Configure default route, static route to the Router. |
| 6. | Configure RIP routing Protocol in Routers. |
| 7. | Configure OSPF routing protocol. |
| 8. | To construct a VLAN and make the PC’s communicate among a VLAN. |
| 9. | To construct a WLAN and make the nodes communicate wirelessly. |
| 10. | Demonstrate the TTL/ Life of a Packet. |
| 11. | To understand the operation of TELNET by accessing the router in server room from a PC in IT office. |
| 12. | To construct simple LAN and understand the concept and operation of Address Resolution Protocol (ARP). |

| PART – B |  |
| --- | --- |
| Serial No. | Name of Expirement |
| 1. | Write a program for congestion control using Leaky bucket algorithm. |
| 2. | Using TCP/IP sockets, write a client-server program to make client sending the file name and the server to send back the contents of the requested file if present. |
| 3. | Using UDP sockets, write a client-server program to make client sending the file name and the server to send back the contents of the requested file if present. |
| 4. | Write a program for error detecting code using CRC-CCITT (16-bits). |

PART - A

Program 1: Create a topology and simulate sending a simple PDU from source to destination using hub and switch as connecting devices and demonstrate ping message.

Network diagram:

Configuration:

| DEVICE | IP ADDRESS | SUBNET MASK |
| --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 |
| PC1 | 192.168.1.2 | 255.255.255.0 |
| LAPTOP0 | 192.168.1.3 | 255.255.255.0 |
| PC2 | 192.168.1.4 | 255.255.255.0 |
| LAPTOP1 | 192.168.1.5 | 255.255.255.0 |
| PC3 | 192.168.1.6 | 255.255.255.0 |

Ping message:
HUB:

SWITCH

Program 2: Configure DHCP within a LAN and outside LAN.

Network diagram:

Configuration:

Server2 IP- 192.168.10.2

Router configuration

All PCs IP is set to DHCP

Ex PC4

Program 3: Configure Web Server, DNS within a LAN.

Network diagram:

Configuration:

| Device | Ip address | Dns address |
| --- | --- | --- |
| PC0 | 192.168.10.1 | 192.168.10.5 |
| PC1 | 192.168.10.2 | 192.168.10.5 |
| Web server | 192.168.10.6 | 192.168.10.5 |
| DNS server | 192.168.10.5 | 192.168.10.5 |

Program 4: Configure IP address to routers in packet tracer. Explore the following messages: ping responses, destination unreachable, request timed out, reply.

Network diagram:

Configuration:

| Device | Ip address |
| --- | --- |
| PC4 | 192.168.20.2 |
| PC5 | 192.168.20.3 |
| PC2 | 192.168.30.10 |
| PC3 | 192.168.30.11 |

Request timed out when default gateway not set

Ping successful

Destination unreachable when an ip address not in network given

Program 5: Configure default route, static route to the Router.

Network diagram:

Configuration:

| DEVICE | IP | DEFAULT GATEWAY |
| --- | --- | --- |
| PC0 | 192.168.10.10 | 192.168.10.1 |
| PC1 | 192.168.20.10 | 192.168.20.1 |
| PC2 | 192.168.30.10 | 192.168.30.1 |

R1: FA0/0-192.168.10.1 , 255.255.255.0

SE2/0- 172.16.1.1 , 255.255.255.252

R2: FA0/0- 192.168.20.1, 255.255.255.0

SE2/0- 172.16.1.2, 255.255.255.252

SE3/0- 172.16.2.1, 255.255.255.252

R3: SE3/0- 17.16.2.2, 255.255.255.252

FA0/0- 192.168.30.1, 255.255.255.0

Program 6: Configure RIP routing Protocol in Routers.

Network diagram:

Configuration:

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | NIC | 192.168.20.2 | 255.255.255.0 | 192.168.1.1 |
| R1 | FastEthernet0/0 | 192.168.20.1 | 255.255.255.0 | — |
| R1 | Serial2/0 | 10.10.0.2 | 255.255.255.252 | — |
| R2 | Serial2/0 | 10.10.0.3 | 255.255.255.252 | — |
| R2 | FastEthernet0/0 | 192.168.10.1 | 255.255.255.0 | — |
| PC1 | NIC | 192.168.10.2 | 255.255.255.0 | 192.168.2.1 |

Program 7: Configure OSPF routing protocol.

Network diagram:

Configuration:

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.55.2 | 255.255.255.0 | 192.168.55.1 |
| PC1 | 192.168.44.2 | 255.255.255.0 | 192.168.44.1 |
| PC2 | 10.10.22.2 | 255.255.255.0 | 10.10.22.1 |

R3:

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| GigabitEthernet0/0 | 192.168.55.1 | 255.255.255.0 |
| GigabitEthernet0/1 | 176.16.10.1 | 255.255.255.0 |

R4:

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| GigabitEthernet0/0 | 192.168.44.1 | 255.255.255.0 |
| GigabitEthernet0/1 | 192.168.55.0 | 255.255.255.0 |

R5:

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| GigabitEthernet0/0 | 176.16.10.2 | 255.255.255.0 |
| GigabitEthernet0/1 | 10.10.22.1 | 255.255.255.0 |

Program 8: To construct a VLAN and make the PC’s communicate among a VLAN.

Network diagram:

Configuration:

Program 9: To construct a WLAN and make the nodes communicate wirelessly.

Network diagram:

Configuration:

Program 10: Demonstrate the TTL/ Life of a Packet.

Network diagram:

Configuration:

Program 11: To understand the operation of TELNET by accessing the router in server room from a PC in IT office.

Network diagram:

Configuration:

Program 12: To construct simple LAN and understand the concept and operation of Address Resolution Protocol (ARP).

Network diagram:

Configuration:

Server0-192.168.11.4

Pc2-192.168.11.1

Pc4-192.168.11.3

Pc3-192.168.11.2

DNS – 192.168.11.4

PART - B

Program 1: Write a program for congestion control using Leaky bucket algorithm.

Code:

#include <stdio.h>

int min(int x, int y) {

return (x < y) ? x : y;

}

int main() {

int cap, process, nsec;

int inp[25];

int count = 0, drop = 0, mini;

printf("Enter the bucket size:\n");

scanf("%d", &cap);

printf("Enter the processing rate:\n");

scanf("%d", &process);

printf("Enter the number of seconds you want to simulate:\n");

scanf("%d", &nsec);

for(int i = 0; i < nsec; i++) {

printf("Enter the size of the packet entering at %d sec:\n", i+1);

scanf("%d", &inp[i]);

}

printf("\nSecond | Packet Recvd | Packet Sent | Packet Left | Dropped\n");

printf("---------------------------------------------------------------\n");

// Main simulation for given seconds

for(int i = 0; i < nsec; i++) {

count += inp[i];  // packets received

if(count > cap) {

drop = count - cap;

count = cap;

}

mini = min(count, process); // process packets

count -= mini;

printf("%6d | %12d | %11d | %11d | %7d\n", i+1, inp[i], mini, count, drop);

drop = 0;

}

// Flush the remaining packets after time ends

int i = nsec;

while(count > 0) {

if(count > cap) {

drop = count - cap;

count = cap;

}

mini = min(count, process);

count -= mini;

printf("%6d | %12d | %11d | %11d | %7d\n", i+1, 0, mini, count, drop);

drop = 0;

i++;

}

return 0;

}

Output:

Program 2: Using TCP/IP sockets, write a client-server program to make client sending the file name and the server to send back the contents of the requested file if present.

Code:

SERVER CODE (Server.c)

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <netinet/in.h>

int main(int argc, char *argv[])

{

int sockfd, newsockfd, portno;

char buffer[256], data[20000];

struct sockaddr_in serv, cli;

socklen_t len;

FILE *fp;

if (argc < 2)

{

printf("Error: No port number provided\n");

printf("Usage: ./server <portno>\n");

exit(1);

}

sockfd = socket(AF_INET, SOCK_STREAM, 0);

serv.sin_family = AF_INET;

serv.sin_addr.s_addr = INADDR_ANY;

serv.sin_port = htons(atoi(argv[1]));

bind(sockfd, (struct sockaddr *)&serv, sizeof(serv));

listen(sockfd, 5);

len = sizeof(cli);

printf("Server waiting for connection...\n");

newsockfd = accept(sockfd, (struct sockaddr *)&cli, &len);

bzero(buffer, 256);

read(newsockfd, buffer, 255);

printf("Server received filename: %s\n", buffer);

fp = fopen(buffer, "r");

if (fp == NULL)

{

write(newsockfd, "File not found", 14);

printf("File not found\n");

}

else

{

printf("File found. Sending data...\n");

bzero(data, 20000);

while (fgets(buffer, 256, fp) != NULL)

{

strcat(data, buffer);

}

write(newsockfd, data, strlen(data));

printf("File sent successfully\n");

fclose(fp);

}

close(newsockfd);

close(sockfd);

return 0;

}

CLIENT CODE (Client.c)

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <netinet/in.h>

int main(int argc, char *argv[])

{

int sockfd;

char filename[256], data[20000];

struct sockaddr_in serv;

if (argc < 2)

{

printf("Error: Port number missing\n");

printf("Usage: ./client <portno>\n");

exit(1);

}

sockfd = socket(AF_INET, SOCK_STREAM, 0);

serv.sin_family = AF_INET;

serv.sin_port = htons(atoi(argv[1]));

serv.sin_addr.s_addr = INADDR_ANY;

connect(sockfd, (struct sockaddr *)&serv, sizeof(serv));

printf("Enter file name: ");

scanf("%s", filename);

write(sockfd, filename, strlen(filename));

bzero(data, 20000);

read(sockfd, data, sizeof(data));

printf("\n--- File Content ---\n");

printf("%s\n", data);

printf("--------------------\n");

close(sockfd);

return 0;

}

Output:

Program 3: Using UDP sockets, write a client-server program to make client sending the file name and the server to send back the contents of the requested file if present.

Code:

Server side: udp_server.c

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <netinet/in.h>

int main()

{

int sockfd, len;

char filename[256], buffer[2000], filedata[20000];

struct sockaddr_in serv, cli;

FILE *fp;

sockfd = socket(AF_INET, SOCK_DGRAM, 0);

serv.sin_family = AF_INET;

serv.sin_port = htons(7777);     // Fixed port

serv.sin_addr.s_addr = INADDR_ANY;

bind(sockfd, (struct sockaddr *)&serv, sizeof(serv));

len = sizeof(cli);

printf("UDP Server waiting...\n");

bzero(filename, 256);

recvfrom(sockfd, filename, sizeof(filename), 0,

(struct sockaddr *)&cli, &len);

printf("Server received filename: %s\n", filename);

fp = fopen(filename, "r");

if (fp == NULL)

{

strcpy(buffer, "File not found");

sendto(sockfd, buffer, strlen(buffer), 0,

(struct sockaddr *)&cli, len);

printf("File not found\n");

}

else

{

bzero(filedata, 20000);

while (fgets(buffer, 2000, fp) != NULL)

{

strcat(filedata, buffer);

}

sendto(sockfd, filedata, strlen(filedata), 0,

(struct sockaddr *)&cli, len);

printf("File sent successfully\n");

fclose(fp);

}

close(sockfd);

return 0;

}

CLIENT CODE (udp_client.c)

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <netinet/in.h>

int main()

{

int sockfd, len;

char filename[256], buffer[20000];

struct sockaddr_in serv;

sockfd = socket(AF_INET, SOCK_DGRAM, 0);

serv.sin_family = AF_INET;

serv.sin_port = htons(7777);

serv.sin_addr.s_addr = INADDR_ANY;

printf("Enter file name: ");

scanf("%s", filename);

sendto(sockfd, filename, strlen(filename), 0,

(struct sockaddr *)&serv, sizeof(serv));

len = sizeof(serv);

bzero(buffer, 20000);

recvfrom(sockfd, buffer, sizeof(buffer), 0,

(struct sockaddr *)&serv, &len);

printf("\n--- File Content ---\n");

printf("%s\n", buffer);

printf("--------------------\n");

close(sockfd);

return 0;

}

Output:

Program 4: Write a program for error detecting code using CRC-CCITT (16-bits).

Code:

#include <stdio.h>

#include <string.h>

#include <stdlib.h>

int main() {

char rem[50], a[80], s[80], c, msj[50], gen[30];

int i, genlen, t, j, flag = 0, k, n;

printf("Enter the generator polynomial (example: 10001000000100001 for CRC-CCITT):\n");

gets(gen);

printf("Generator polynomial is CRC-CCITT: %s\n", gen);

genlen = strlen(gen);

k = genlen - 1;

printf("Enter the message:\n");

n = 0;

while ((c = getchar()) != '\n' && c != EOF) {

msj[n++] = c;

}

msj[n] = '\0';

// Copy message and append k zeros

for (i = 0; i < n; i++)

a[i] = msj[i];

for (i = 0; i < k; i++)

a[n + i] = '0';

a[n + k] = '\0';

printf("\nMessage polynomial appended with zeros:\n");

puts(a);

// Division to get remainder

for (i = 0; i < n; i++) {

if (a[i] == '1') {

t = i;

for (j = 0; j <= k; j++) {

a[t] = (a[t] == gen[j]) ? '0' : '1';

t++;

}

}

}

// Extract remainder

for (i = 0; i < k; i++)

rem[i] = a[n + i];

rem[k] = '\0';

printf("The checksum (remainder) is:\n");

puts(rem);

printf("\nMessage with checksum appended:\n");

for (i = 0; i < n; i++)

a[i] = msj[i];

for (i = 0; i < k; i++)

a[n + i] = rem[i];

a[n + k] = '\0';

puts(a);

// Receiver side

printf("\nEnter the received message:\n");

n = 0;

while ((c = getchar()) != '\n' && c != EOF) {

s[n++] = c;

}

s[n] = '\0';

// Re-division at receiver

for (i = 0; i < n; i++) {

if (s[i] == '1') {

t = i;

for (j = 0; j <= k; j++, t++) {

s[t] = (s[t] == gen[j]) ? '0' : '1';

}

}

}

// Check remainder at receiver

for (i = 0; i < k; i++)

rem[i] = s[n + i];

rem[k] = '\0';

for (i = 0; i < k; i++) {

if (rem[i] == '1')

flag = 1;

}

if (flag == 0)

printf("Received polynomial is ERROR-FREE\n");

else

printf("Received polynomial contains ERROR\n");

return 0;

}

Output: